Savitribai Phule Pune University

University Practical Examination — Mar/April 2026

F.Y.B.Sc.(IT) Sem-II (NEP Pattern)

IT154MJP — Lab Course on Advanced Networking

Complete Answers to All 20 Practical Slips

SLIP 1

## Q.1) Identify the IP Address of a Workstation

Steps (Windows:)

- Press Win + R → type cmd → press Enter

- Type ipconfig and press Enter

- Note the IPv4 Address, Subnet Mask, and Default Gateway

C:\> ipconfig
Ethernet adapter:
  IPv4 Address   : 192.168.1.105
  Subnet Mask    : 255.255.255.0
  Default Gateway: 192.168.1.1

Steps (Linux/Mac):

ifconfig        # older systems
ip addr show    # newer Linux

## Q.2 a) Test Connectivity of Default Gateway (192.168.1.1) using Ping

ping 192.168.1.1

Pinging 192.168.1.1 with 32 bytes of data:
Reply from 192.168.1.1: bytes=32 time=1ms TTL=64
Packets: Sent=4, Received=4, Lost=0 (0% loss)

## Q.2 b) Documentation of Process

| Step | Action | Result |
| --- | --- | --- |
| 1 | Open CMD | Command Prompt launched |
| 2 | Run ipconfig | Found IP: 192.168.1.105 |
| 3 | Run ping 192.168.1.1 | 4/4 packets received, 0% loss |
| 4 | Issue: Request Timed Out | Solution: Disable firewall temporarily |
| 5 | Issue: Wrong IP configured | Solution: Reconfigure via Network Adapter settings |

Common Issues & Solutions:

- Request Timed Out → Check firewall, verify gateway IP, ensure cable connected

- Destination Host Unreachable → Check if IP is in same subnet as gateway

- General Failure → Reinstall network driver or check NIC

SLIP 2

## Q.1) Why Traditional IP Cannot Be Used in Mobile Networks & Differences

Why Traditional IP Fails:

- In traditional IP, a device's IP is tied to its physical network location

- When a mobile device moves, its IP changes → ongoing connections break

- Routers route packets based on IP — if IP changes mid-session, session dies

- Traditional IP has no mobility concept — no mechanism to forward packets to new location

| Feature | Traditional IP | Mobile IP |
| --- | --- | --- |
| Mobility Support | No | Yes |
| IP on Movement | Changes (breaks connection) | Permanent Home IP retained |
| Agents Required | No | Home Agent & Foreign Agent |
| Tunneling | Not used | IP-in-IP tunneling |
| Care-of Address | Not used | Used in foreign network |
| Routing | Direct | Triangular via HA |
| Connection Continuity | Lost on movement | Maintained |

## Q.2) Configure IPv4, IPv6 and Learn Quality, Security and Other Services

IPv4 Configuration (Windows):

- Control Panel → Network & Sharing Center → Change Adapter Settings

- Right-click adapter → Properties → IPv4 → Properties

- Enter: IP: 192.168.1.10 | Subnet: 255.255.255.0 | Gateway: 192.168.1.1 | DNS: 8.8.8.8

IPv6 Configuration:

- Same path → IPv6 → Properties → Manual

- IP: 2001:db8::1 | Prefix: 64 | Gateway: 2001:db8::fffe

QoS: Prioritizes traffic (voice/video over bulk data). Set via router admin → QoS settings.

Security: WPA2/WPA3 encryption, MAC Address Filtering, Firewall rules.

SLIP 3

## Q.1) Differentiate Functionalities of Foreign Agent & Home Agent

| Feature | Home Agent (HA) | Foreign Agent (FA) |
| --- | --- | --- |
| Location | MN's home network | Visited/foreign network |
| Function | Maintains MN location info | Provides temporary services to MN |
| IP Management | Holds permanent Home IP | Provides Care-of-Address (COA) |
| Packet Handling | Intercepts & tunnels to FA | Decapsulates & delivers to MN |
| Registration | Receives registration from MN | Forwards MN's registration to HA |
| Binding Table | Maintains MN's COA binding | Maintains visitor list |

Summary: HA = MN's permanent home (always knows where MN is). FA = Temporary host in visited network.

## Q.2) Find Local IPv4 & Subnet Mask; Calculate Subnets and Hosts

Step 1: Run ipconfig → Example: IP: 192.168.1.105, Mask: 255.255.255.0 (/24)

Calculations for /24:

| Parameter | Calculation | Value |
| --- | --- | --- |
| Host bits | 32 - 24 | 8 |
| Hosts per subnet | 2^8 - 2 | 254 usable hosts |
| Network Address | — | 192.168.1.0 |
| Broadcast | — | 192.168.1.255 |
| Usable Range | — | 192.168.1.1 to 192.168.1.254 |

SLIP 4

## Q.1) Steps to Configure Mobile IP on Android Mobile Device

- Go to Settings → Wi-Fi

- Long-press your network → Modify Network / Manage Network Settings

- Tap Show Advanced Options

- Change IP Settings from DHCP to Static

| Field | Value |
| --- | --- |
| IP Address | 192.168.1.130 |
| Gateway | 192.168.1.1 |
| Network Prefix Length | 24 |
| DNS 1 | 8.8.8.8 |
| DNS 2 | 8.8.4.4 |

- Tap Save/Connect

- Verify: Settings → About Phone → Status → IP Address

Note: This configures a static IP on mobile, mimicking Mobile IP behavior on a local network.

## Q.2) Configure IPv4, IPv6 and Services

Refer to Slip 2 Q.2 for detailed IPv4/IPv6 configuration steps, QoS, and Security services.

SLIP 5

## Q.1) Explain the Handover Process When Mobile Device Moves Networks

| Phase | Step | Description |
| --- | --- | --- |
| 1 | Detection | MN detects signal loss from old FA; detects new FA beacon |
| 2 | Agent Discovery | MN listens for Agent Advertisements; sends Agent Solicitation if none |
| 3 | COA Acquisition | MN obtains new Care-of-Address from new FA or via DHCP |
| 4 | Registration | MN sends Registration Request → FA → HA; HA updates binding table |
| 5 | Tunneling | HA tunnels (IP-in-IP) all packets to new COA; FA delivers to MN |
| 6 | Continuity | MN's Home IP unchanged; all active sessions remain intact |

## Q.2) Configure Mobile IP on Android — IP: 192.168.1.130

- Settings → Wi-Fi → Long press network → Modify → Advanced Options

- IP Settings → Static

- IP Address: 192.168.1.130 | Gateway: 192.168.1.1 | Prefix: 24

- DNS: 8.8.8.8 → Save

- Verify: ping 192.168.1.1 from another device

SLIP 6

## Q.1 a) Key Mechanisms in Mobile IP

| Mechanism | Description |
| --- | --- |
| Agent Discovery | MN finds HA or FA via ICMP Router Advertisement |
| Registration | MN registers COA with HA; HA updates binding table |
| IP-in-IP Tunneling | HA encapsulates packets and sends to COA (FA) |
| Triangle Routing | Sender → HA → FA → MN (indirect path) |
| De-encapsulation | FA strips outer IP header; delivers packet to MN |
| Route Optimization | Correspondent node caches COA for direct delivery (optional) |

## Q.1 b) Care-Of-Address (COA) — Definition and Types

Definition: COA is the temporary IP address assigned to a mobile node in the foreign network. It represents the MN's current topological location.

| Type | Description | Feature |
| --- | --- | --- |
| Foreign Agent COA | IP address of the FA itself | Multiple MNs can share it |
| Co-located COA | Unique IP assigned to MN's interface (DHCP/manual) | MN independence; no FA needed |

## Q.2) Calculate Subnets and Hosts for 192.168.0.0/28

Given: 192.168.0.0/28 | Subnet Mask: 255.255.255.240

| Parameter | Calculation | Value |
| --- | --- | --- |
| Subnet bits borrowed | 28 - 24 | 4 |
| Number of Subnets | 2^4 | 16 |
| Host bits | 32 - 28 | 4 |
| Hosts per Subnet | 2^4 - 2 | 14 usable hosts |
| Block Size | 256 - 240 | 16 |

| Subnet | Network Address | Usable Range | Broadcast |
| --- | --- | --- | --- |
| 1 | 192.168.0.0 | 192.168.0.1 – .14 | 192.168.0.15 |
| 2 | 192.168.0.16 | 192.168.0.17 – .30 | 192.168.0.31 |
| 3 | 192.168.0.32 | 192.168.0.33 – .46 | 192.168.0.47 |
| ... | ... | ... | ... |
| 16 | 192.168.0.240 | 192.168.0.241 – .254 | 192.168.0.255 |

SLIP 7

## Q.1) Identify the IP Address of a Workstation

Run ipconfig (Windows) or ip addr show (Linux) to find the IPv4 Address, Subnet Mask, and Default Gateway. Refer to Slip 1 Q.1 for full steps.

## Q.2) Subnet Calculations for 192.168.10.0/26 (Using Spreadsheet)

Given: 192.168.10.0/26 | Subnet Mask: 255.255.255.192

| Parameter | Calculation | Value |
| --- | --- | --- |
| Subnet bits | 26 - 24 | 2 |
| Number of Subnets | 2^2 | 4 |
| Host bits | 32 - 26 | 6 |
| Hosts per Subnet | 2^6 - 2 | 62 |
| Block Size | 256 - 192 | 64 |

| Subnet # | Network Address | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- |
| 1 | 192.168.10.0 | 192.168.10.1 | 192.168.10.62 | 192.168.10.63 |
| 2 | 192.168.10.64 | 192.168.10.65 | 192.168.10.126 | 192.168.10.127 |
| 3 | 192.168.10.128 | 192.168.10.129 | 192.168.10.190 | 192.168.10.191 |
| 4 | 192.168.10.192 | 192.168.10.193 | 192.168.10.254 | 192.168.10.255 |

Excel Formula hints: First Host = Network + 1 | Broadcast = Network + 63 | Last Host = Broadcast - 1

SLIP 8

## Q.1) Default IP Address & SSID of Lab Router

| Brand | Default IP | Default SSID |
| --- | --- | --- |
| TP-Link | 192.168.0.1 or 192.168.1.1 | TP-Link_XXXX |
| D-Link | 192.168.0.1 | dlink |
| Netgear | 192.168.1.1 | NETGEAR |
| Cisco | 192.168.1.1 | varies |

To find SSID: Check router label (bottom sticker) OR click Wi-Fi icon in Windows → connected network name OR run: netsh wlan show interfaces

## Q.2) Convert CIDR to Subnet Mask — 192.168.1.0/26

Given: /26 → first 26 bits are 1s, remaining 6 bits are 0s

Binary: 11111111.11111111.11111111.11000000

| Parameter | Value |
| --- | --- |
| IP Address | 192.168.1.0 |
| CIDR Notation | /26 |
| Subnet Mask (Decimal) | 255.255.255.192 |
| Block Size | 64 |
| Number of Subnets | 4 |
| Hosts per Subnet | 62 |

SLIP 9

## Q.1) IP Address Range Assigned to Network

Steps: Run ipconfig → note IP (e.g., 192.168.1.105) and mask (255.255.255.0 = /24)

| Parameter | Value |
| --- | --- |
| Network Address | 192.168.1.0 |
| Broadcast Address | 192.168.1.255 |
| Usable IP Range | 192.168.1.1 to 192.168.1.254 |
| Total Usable Hosts | 254 |

## Q.2) Difference Between Dynamic and Static IPv4 Addressing

| Feature | Static IPv4 | Dynamic IPv4 (DHCP) |
| --- | --- | --- |
| Assignment | Manually configured | Auto-assigned by DHCP server |
| IP Consistency | Always the same | Changes each connection |
| Configuration Effort | High | Low (automatic) |
| Use Case | Servers, printers, routers | Client PCs, mobile devices |
| IP Conflict Risk | Higher (human error) | Lower (DHCP manages it) |
| Scalability | Poor for large networks | Excellent |
| Troubleshooting | Easier to pinpoint | Harder (IP changes) |

SLIP 10

## Q.1) Maximum Range of IP Addresses

IPv4: 32-bit → Total = 2^32 = 4,294,967,296 (~4.3 billion) | Range: 0.0.0.0 to 255.255.255.255

| Range | Purpose |
| --- | --- |
| 0.0.0.0 – 0.255.255.255 | "This" network |
| 10.0.0.0 – 10.255.255.255 | Private (Class A) |
| 127.0.0.0 – 127.255.255.255 | Loopback |
| 172.16.0.0 – 172.31.255.255 | Private (Class B) |
| 192.168.0.0 – 192.168.255.255 | Private (Class C) |
| 255.255.255.255 | Limited Broadcast |

IPv6: 2^128 = approximately 340 undecillion addresses

## Q.2) Configure Router with Dynamic IPv4 and Write Assigned IP

- Connect PC to router via LAN or Wi-Fi

- Open browser → go to 192.168.1.1 → Login (admin/admin)

- Go to DHCP Settings → Enable DHCP Server

- Set range: 192.168.1.100 to 192.168.1.200 | Subnet: 255.255.255.0 | GW: 192.168.1.1

- Save and Reboot router

- On client PC: Set IP to Obtain automatically (DHCP)

- Run ipconfig → note the dynamically assigned IP address

SLIP 11

## Q.1) Explain IPv6 Addressing with Suitable Examples

IPv6 is a 128-bit address written as 8 groups of 4 hex digits separated by colons (:)

Full Example: 2001:0db8:85a3:0000:0000:8a2e:0370:7334

Simplification Rules:

- Leading zeros in a group can be omitted: 0db8 → db8

- Consecutive all-zero groups → replaced by :: (only once allowed)

Simplified: 2001:0db8:0000:0000:0000:0000:0000:0001  →  2001:db8::1

| Type | Example | Purpose |
| --- | --- | --- |
| Unicast | 2001:db8::1 | Single interface |
| Multicast | ff02::1 | Group of interfaces |
| Loopback | ::1 | Self (like 127.0.0.1) |
| Link-local | fe80::1 | Same link only |
| Global Unicast | 2001::/16 | Public internet |
| Anycast | 2001:db8::1/48 | Nearest of a group |

## Q.2) Configure Routers and Hosts with IPv6; Test Connectivity

Router Config (Cisco Packet Tracer):

Router> enable
Router# config t
Router(config)# ipv6 unicast-routing
Router(config)# interface GigabitEthernet0/0
Router(config-if)# ipv6 address 2001:db8:1::1/64
Router(config-if)# no shutdown

Host Config (Windows): Network Adapter → IPv6 → Manual → IP: 2001:db8:1::10, Prefix: 64, GW: 2001:db8:1::1

Test Connectivity:

ping -6 2001:db8:1::1    (Windows)
ping6 2001:db8:1::1     (Linux)

SLIP 12

## Q.1) How to Subnet an IPv6 Network into Smaller Subnets

IPv6 Subnetting: ISP gives /48, site uses bits 49-64 for subnets → up to 65,536 /64 subnets. Each subnet = /64.

Example: Subnet 2001:db8:abcd::/48 into /52 subnets

| Parameter | Value |
| --- | --- |
| Base Prefix | /48 |
| New Prefix | /52 |
| Bits for subnetting | 52 - 48 = 4 |
| Number of Subnets | 2^4 = 16 |
| Increment (hex) | 0x1000 in 4th group |

| Subnet # | Address |
| --- | --- |
| 1 | 2001:db8:abcd:0000::/52 |
| 2 | 2001:db8:abcd:1000::/52 |
| 3 | 2001:db8:abcd:2000::/52 |
| ... | ... |
| 16 | 2001:db8:abcd:f000::/52 |

Steps: 1) Identify prefix  2) Choose new prefix  3) Calculate extra bits  4) Increment in hex

## Q.2) Configure IPv4, IPv6 and Services

Refer to Slip 2 Q.2 for full IPv4/IPv6 configuration steps, QoS, and Security settings.

SLIP 13

## Q.1) Connect Two Systems to Router with Dynamic IPv4 and Check Connections

- Connect PC1 and PC2 to router via LAN cables (or Wi-Fi)

- Set both PCs to DHCP: Control Panel → Network → Adapter → IPv4 → Obtain IP automatically

- Verify IPs: PC1 runs ipconfig → e.g., 192.168.1.101 | PC2 → 192.168.1.102

Test Connectivity:

From PC1: ping 192.168.1.102
From PC2: ping 192.168.1.101

Expected Result: Replies received = connection is working (0% packet loss)

## Q.2) Subnet Calculation — Base Prefix /32, Need 65536 Subnets (IPv6)

| Parameter | Calculation | Value |
| --- | --- | --- |
| Subnets Required | — | 65,536 = 2^16 |
| Subnet bits borrowed | log2(65536) | 16 |
| New Subnet Prefix | 32 + 16 | /48 |
| Host bits | 128 - 48 | 80 |
| Total addresses/subnet | 2^80 | ~1.2 x 10^24 |
| Usable hosts/subnet | 2^80 - 2 | Virtually unlimited |

SLIP 14

## Q.1) Difference Between IPv4 and IPv6

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Address Length | 32-bit | 128-bit |
| Format | Dotted decimal (192.168.1.1) | Hex with colons (2001:db8::1) |
| Total Addresses | ~4.3 billion | ~3.4 x 10^38 |
| Header Size | 20-60 bytes (variable) | 40 bytes (fixed) |
| NAT Required | Yes (address exhaustion) | No |
| Security (IPSec) | Optional | Mandatory |
| Auto-configuration | Requires DHCP | SLAAC available |
| Broadcast | Supported | Replaced by Multicast |
| Fragmentation | By routers and sender | Only by sender |
| QoS | Limited (ToS field) | Better (Flow Label field) |
| DNS Records | A records | AAAA records |

## Q.2) Configure IPv4, IPv6 and Services

Refer to Slip 2 Q.2 for detailed IPv4/IPv6 configuration steps and services.

SLIP 15

## Q.1) Home Network and Foreign Network in Mobile IP

| Aspect | Home Network | Foreign Network |
| --- | --- | --- |
| Definition | Where MN is originally registered | Any network MN visits |
| Agent Present | Home Agent (HA) | Foreign Agent (FA) |
| IP Role | MN holds permanent Home IP here | FA provides temporary COA |
| Packet Handling | HA intercepts & tunnels to FA | FA delivers packets to MN |
| Example | Phone's home cellular network | Hotel or airport Wi-Fi |

Flow: Correspondent → HA (home network) → [tunnel] → FA (foreign network) → MN

## Q.2) Compare IPv4 and IPv6 Subnetting

| Aspect | IPv4 Subnetting | IPv6 Subnetting |
| --- | --- | --- |
| Notation | Dotted decimal | Hexadecimal with colons |
| Subnet Mask | 255.255.255.0 style | Prefix length only (/64) |
| CIDR Example | 192.168.1.0/24 | 2001:db8::/32 |
| Host Subnet | Typically /24 to /30 | Always /64 |
| Subnet Formula | 2^n subnets, 2^h-2 hosts | 2^n subnets, 2^64-2 hosts |
| Broadcast | Uses broadcast address | No broadcast (uses multicast) |
| Auto-config | Requires DHCP | SLAAC available |

SLIP 16

## Q.1) Connect Two Systems to Router with Dynamic IPv4

Refer to Slip 13 Q.1 for complete steps to connect two PCs via DHCP and test ping connectivity.

## Q.2) Components of Mobile IP — HA, FA, and MN

1. Mobile Node (MN):

- The mobile device (laptop, phone) that moves between networks

- Has a permanent Home Address that never changes

- Registers with HA when visiting a foreign network

2. Home Agent (HA):

- A router in the MN's home network

- Maintains binding table: maps MN's Home IP → current COA

- Intercepts packets for MN and tunnels them to FA

- Authenticates MN's registration requests

3. Foreign Agent (FA):

- A router in the visited network

- Sends Agent Advertisements to announce presence

- Provides Care-of-Address (COA) to MN

- Receives tunneled packets from HA and forwards to MN

- Maintains a visitor list of all registered MNs

Interaction Flow: MN arrives → FA advertises → MN registers with FA → FA forwards to HA → HA updates binding → Packets: CN → HA → (tunnel) → FA → MN

SLIP 17

## Q.1) Identify IP Address of a Workstation

Use ipconfig (Windows) or ip addr show (Linux) to identify IPv4 address, subnet mask, and default gateway. Refer to Slip 1 Q.1 for full steps.

## Q.2) Define Mobile IP and How It Enables Seamless Mobility

Definition: Mobile IP (RFC 3344 for IPv4, RFC 3775 for IPv6) is an IETF standard that allows mobile devices to move between networks while maintaining a permanent IP address, ensuring continuous communication without dropping active sessions.

| Feature | Explanation |
| --- | --- |
| Permanent Address | MN always keeps Home IP → applications see no change |
| Location Transparency | HA hides MN's actual location from correspondent node |
| Registration Protocol | MN notifies HA of new COA when it moves to foreign network |
| Tunneling | HA encapsulates packets to new location → no session interruption |
| Handover Support | MN re-registers with new FA; HA updates binding → sessions continue |
| Protocol Independence | Works above IP layer; applications need no modification |

Key Benefit: A video call started at home continues uninterrupted as the device moves to a foreign network — the application never knows the underlying IP path changed.

SLIP 18

## Q.1) Identify Class of IP Address and Configure on Workstation

| Class | Range | Default Mask | Use |
| --- | --- | --- | --- |
| A | 1.0.0.0 – 126.255.255.255 | 255.0.0.0 (/8) | Large networks |
| B | 128.0.0.0 – 191.255.255.255 | 255.255.0.0 (/16) | Medium networks |
| C | 192.0.0.0 – 223.255.255.255 | 255.255.255.0 (/24) | Small networks |
| D | 224.0.0.0 – 239.255.255.255 | N/A | Multicast |
| E | 240.0.0.0 – 255.255.255.255 | N/A | Research/Reserved |

Example: 192.168.1.10 → starts with 192 → Class C

Configure IP on Workstation (Windows):

- Control Panel → Network & Sharing Center → Change Adapter Settings

- Right-click → Properties → Internet Protocol Version 4 → Properties

- Select 'Use the following IP address'

- IP: 192.168.1.10 | Mask: 255.255.255.0 | Gateway: 192.168.1.1 → OK

## Q.2) Configure IPv4, IPv6 and Services

Refer to Slip 2 Q.2 for full configuration steps.

SLIP 19

## Q.1) Connect Two Systems with Dynamic IPv4 and Check Connections

Refer to Slip 13 Q.1 for detailed steps to connect two PCs, obtain DHCP IPs, and test connectivity with ping.

## Q.2) Steps in Handover Process When Mobile Device Moves Networks (Detailed)

| Phase | Description |
| --- | --- |
| 1. Movement Detection | MN monitors signal from current FA. Detects degraded signal / loss of Agent Advertisement. Identifies new FA via stronger advertisements. |
| 2. COA Acquisition | FA-COA: MN uses FA's IP as COA (most common). Co-located COA: MN gets new IP via DHCP. |
| 3. Registration | MN sends Registration Request to new FA (contains: Home IP, HA IP, new COA, lifetime). FA forwards to HA. |
| 4. HA Updates Binding | HA verifies MN identity. Updates Binding Cache: Home IP → new COA. Sends Registration Reply to FA → MN. |
| 5. Packet Redirection | All packets for MN's Home IP now tunneled to new COA. Old FA discards further packets. |
| 6. Deregistration | MN may send deregistration to old FA to free resources (optional). |
| Result | Seamless handover — TCP connections remain active throughout the entire process. |

SLIP 20

## Q.1) How to Subnet an IPv6 Network into Smaller Subnets

Refer to Slip 12 Q.1 for complete IPv6 subnetting explanation, example with /48 → /52, and subnet table.

## Q.2) Convert CIDR to Subnet Mask — 192.168.1.0/26

Given: 192.168.1.0/26

Binary: 11111111.11111111.11111111.11000000

| Parameter | Value |
| --- | --- |
| CIDR | /26 |
| Subnet Mask (Binary) | 11111111.11111111.11111111.11000000 |
| Subnet Mask (Decimal) | 255.255.255.192 |
| Block Size | 64 |
| Number of Subnets | 4 |
| Hosts per Subnet | 62 |

| Subnet | Network | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- |
| 1 | 192.168.1.0 | 192.168.1.1 | 192.168.1.62 | 192.168.1.63 |
| 2 | 192.168.1.64 | 192.168.1.65 | 192.168.1.126 | 192.168.1.127 |
| 3 | 192.168.1.128 | 192.168.1.129 | 192.168.1.190 | 192.168.1.191 |
| 4 | 192.168.1.192 | 192.168.1.193 | 192.168.1.254 | 192.168.1.255 |

All 20 Practical Slips Answered | IT154MJP Advanced Networking | F.Y.B.Sc.(IT) Sem-II NEP | SPPU Mar/April 2026