IT154MJP — Advanced Networking

VIVA Questions & Answers

SECTION 1: IP Address Basics

Q1. What is an IP address?

Ans: An IP address is a unique number given to every device on a network. It works like a home address — it tells where to send data. Example: 192.168.1.10

Q2. What command is used to find IP address in Windows?

Ans: ipconfig — Open Command Prompt (cmd) and type ipconfig. It shows your IPv4 address, subnet mask, and default gateway.

Q3. What command is used in Linux to find IP address?

Ans: Use 'ip addr show' (new systems) or 'ifconfig' (old systems).

Q4. What is a subnet mask?

Ans: A subnet mask tells which part of the IP address is the network and which part is the device. Example: 255.255.255.0 means first 3 parts are network, last part is for devices.

Q5. What is a default gateway?

Ans: Default gateway is the router's IP address. When your computer wants to talk to the internet or another network, it first sends the data to the default gateway. Example: 192.168.1.1

Q6. What is the ping command used for?

Ans: ping checks if two devices can talk to each other over a network. Example: 'ping 192.168.1.1' sends test packets and shows if they come back.

Q7. What does 'Request Timed Out' mean in ping?

Ans: It means the device did not reply. Possible reasons: firewall is blocking, the device is off, or the IP address is wrong.

Q8. What does '0% loss' mean in ping result?

Ans: It means all packets were sent and received successfully. The connection is working perfectly.

Q9. What is static IP address?

Ans: A static IP is an address you set manually. It never changes. Used for servers and printers so they always have the same address.

Q10. What is dynamic IP address (DHCP)?

Ans: A dynamic IP is automatically given by a DHCP server. It can change every time the device connects. Used for laptops, phones, and PCs.

SECTION 2: IP Classes & Subnetting

Q11. What are the classes of IP addresses?

Ans: There are 5 classes: A (1-126), B (128-191), C (192-223), D (224-239 for multicast), E (240-255 for research). Most home/office networks use Class C.

| Class | Range | Default Mask | Used For |
| --- | --- | --- | --- |
| A | 1.0.0.0 – 126.x.x.x | 255.0.0.0 | Very large networks |
| B | 128.x – 191.x.x.x | 255.255.0.0 | Medium networks |
| C | 192.x – 223.x.x.x | 255.255.255.0 | Small networks (home/office) |
| D | 224.x – 239.x.x.x | N/A | Multicast |
| E | 240.x – 255.x.x.x | N/A | Research only |

Q12. How do you find the class of an IP address? Give example.

Ans: Look at the first number of the IP. If it starts with 192 → Class C. If 172 → Class B. If 10 → Class A. Example: 192.168.1.5 → Class C.

Q13. What is subnetting?

Ans: Subnetting means dividing a big network into smaller networks (subnets). Like dividing a big building into floors. It saves IP addresses and improves security.

Q14. What is CIDR notation?

Ans: CIDR (Classless Inter-Domain Routing) shows the subnet mask as a number after '/'. Example: 192.168.1.0/24 means the subnet mask has 24 bits set to 1, which equals 255.255.255.0.

Q15. How many hosts can a /24 network have?

Ans: Host bits = 32 - 24 = 8. Total hosts = 2^8 - 2 = 254 usable hosts. (We subtract 2 for network address and broadcast address.)

Q16. Calculate subnets and hosts for 192.168.1.0/26.

Ans: Subnet bits = 26 - 24 = 2. Subnets = 2^2 = 4. Host bits = 32 - 26 = 6. Hosts per subnet = 2^6 - 2 = 62. Subnet mask = 255.255.255.192.

Q17. Calculate subnets and hosts for 192.168.0.0/28.

Ans: Subnet bits = 28 - 24 = 4. Subnets = 2^4 = 16. Host bits = 32 - 28 = 4. Hosts per subnet = 2^4 - 2 = 14. Subnet mask = 255.255.255.240.

Q18. What is a network address?

Ans: Network address is the first IP in a subnet. All host bits are 0. Example: In 192.168.1.0/24, the network address is 192.168.1.0. It identifies the network, not any device.

Q19. What is a broadcast address?

Ans: Broadcast address is the last IP in a subnet. All host bits are 1. Example: In 192.168.1.0/24, broadcast is 192.168.1.255. Data sent here goes to ALL devices in the subnet.

Q20. What is the maximum number of IPv4 addresses?

Ans: IPv4 is 32-bit. Total = 2^32 = 4,294,967,296 (about 4.3 billion) addresses. Range is 0.0.0.0 to 255.255.255.255.

SECTION 3: IPv4 vs IPv6

Q21. What is IPv4?

Ans: IPv4 is version 4 of the Internet Protocol. It uses 32-bit addresses written in dotted decimal format. Example: 192.168.1.1. It has about 4.3 billion addresses.

Q22. What is IPv6?

Ans: IPv6 is version 6 of the Internet Protocol. It uses 128-bit addresses written in hexadecimal with colons. Example: 2001:db8::1. It has 340 undecillion addresses — virtually unlimited!

Q23. Why was IPv6 created?

Ans: IPv4 addresses were running out because of too many internet devices. IPv6 was created to provide more addresses and better features like built-in security.

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Address size | 32-bit | 128-bit |
| Example | 192.168.1.1 | 2001:db8::1 |
| Total addresses | ~4.3 billion | 340 undecillion |
| Header size | Variable (20-60 bytes) | Fixed (40 bytes) |
| Security (IPSec) | Optional | Built-in (mandatory) |
| Broadcast | Yes | No (uses Multicast) |
| Auto-config | Needs DHCP | SLAAC (automatic) |
| NAT needed | Yes | No |

Q24. What is SLAAC in IPv6?

Ans: SLAAC stands for Stateless Address Autoconfiguration. In IPv6, a device can automatically create its own IP address without a DHCP server. It uses the router prefix + device's MAC address.

Q25. What are IPv6 address types?

Ans: Unicast (one device), Multicast (group of devices), Anycast (nearest device in a group). IPv6 has NO broadcast — it uses multicast instead.

Q26. What is a loopback address?

Ans: Loopback address is used to test your own device. IPv4 loopback = 127.0.0.1. IPv6 loopback = ::1. Ping 127.0.0.1 to check if your network card is working.

Q27. How to simplify an IPv6 address?

Ans: Rule 1: Remove leading zeros in each group (0db8 → db8). Rule 2: Replace one group of consecutive all-zero sections with :: (only once). Example: 2001:0000:0000:0001 → 2001::1

Q28. How do you subnet IPv6?

Ans: ISP gives /48 prefix. You use bits 49-64 (16 bits) for subnets. This gives 2^16 = 65,536 subnets. Each subnet is /64. The last 64 bits are for host addresses.

SECTION 4: Mobile IP

Q29. What is Mobile IP?

Ans: Mobile IP is a standard (RFC 3344) that allows a mobile device to keep the same IP address even when it moves from one network to another. So your video call or download does not break when you move.

Q30. Why can't traditional IP be used in mobile networks?

Ans: In traditional IP, your IP address is tied to your location (network). When you move to a new network, your IP changes — this breaks all ongoing connections like calls or downloads. Mobile IP solves this.

Q31. What is a Home Network?

Ans: Home network is the original network where the mobile device is registered. The device has a permanent IP address (Home IP) here. Even when the device travels away, this IP stays the same.

Q32. What is a Foreign Network?

Ans: Foreign network is any other network the mobile device visits. When you go to a hotel or airport and connect to their Wi-Fi, that is a foreign network for your phone.

Q33. What is a Home Agent (HA)?

Ans: Home Agent is a router in the home network. It keeps track of where the mobile device is currently located. When someone sends data to your phone, HA receives it and forwards it to your current location.

Q34. What is a Foreign Agent (FA)?

Ans: Foreign Agent is a router in the visited (foreign) network. It helps the mobile device connect and receive data while away from home. It gives a temporary address (COA) to the device.

Q35. What is a Mobile Node (MN)?

Ans: Mobile Node is the actual mobile device (your phone or laptop) that moves between networks. It keeps its permanent Home IP and communicates using it no matter where it is.

Q36. What is Care-of-Address (COA)?

Ans: COA is a temporary IP address given to the mobile device when it visits a foreign network. It shows the current location of the device. HA uses COA to forward packets to the mobile device.

Q37. What are the types of COA?

Ans: 1) Foreign Agent COA: FA's own IP address is used as COA. Multiple devices can share it. 2) Co-located COA: A unique IP is directly assigned to the mobile device via DHCP in the foreign network.

Q38. What is Agent Discovery in Mobile IP?

Ans: Agent Discovery is how a mobile device finds a Home Agent or Foreign Agent. Agents send special messages called 'Agent Advertisements'. If the device doesn't receive one, it sends an 'Agent Solicitation' to ask.

Q39. What is Registration in Mobile IP?

Ans: Registration is when the mobile device tells the Home Agent its new address (COA). Steps: MN sends Registration Request → FA forwards to HA → HA updates its table → HA sends Registration Reply.

Q40. What is Tunneling in Mobile IP?

Ans: Tunneling is like putting a letter inside another envelope. HA wraps (encapsulates) the original packet in a new packet and sends it to the COA (FA). FA unwraps it and delivers to MN. This is called IP-in-IP tunneling.

Q41. Explain the handover process simply.

Ans: Handover = switching from one network to another without disconnecting. Steps: 1) MN detects weak signal from old network. 2) MN finds new FA. 3) MN gets new COA. 4) MN registers with HA via new FA. 5) HA sends data to new location. 6) Connection continues!

Q42. What is Triangle Routing in Mobile IP?

Ans: Triangle Routing means data takes an indirect path: Sender → Home Agent → Foreign Agent → Mobile Node. It is called triangular because 3 points are involved. This can be slow but it works.

Q43. What is the difference between Home Agent and Foreign Agent?

Ans: HA lives in home network, keeps permanent record of MN's location, and tunnels data to FA. FA lives in visited network, gives temporary address to MN, and receives tunneled data from HA and delivers to MN.

Q44. How to configure Mobile IP on Android?

Ans: Settings → Wi-Fi → Long press network → Modify → Show Advanced Options → IP Settings: Static → Enter IP: 192.168.1.130, Gateway: 192.168.1.1, Prefix: 24, DNS: 8.8.8.8 → Save.

SECTION 5: Router & DHCP

Q45. What is a router?

Ans: A router is a device that connects different networks together and forwards data between them. Your home Wi-Fi router connects your home network to the internet.

Q46. What is DHCP?

Ans: DHCP (Dynamic Host Configuration Protocol) is a service that automatically gives IP addresses to devices. When your phone connects to Wi-Fi, the router's DHCP server gives it an IP address automatically.

Q47. What is SSID?

Ans: SSID (Service Set Identifier) is the name of a Wi-Fi network. It is what you see when you search for Wi-Fi on your phone. Example: 'TP-Link_1234' or 'College_WiFi'.

Q48. What is the default IP of a router?

Ans: Most routers use 192.168.1.1 or 192.168.0.1 as default IP. You can open this in a browser to access the router settings page. Check the sticker on the bottom of the router.

Q49. How to configure a router for DHCP?

Ans: Open browser → go to 192.168.1.1 → Login (admin/admin) → Go to DHCP Settings → Enable DHCP → Set IP range (e.g., 192.168.1.100 to .200) → Save → Restart router.

Q50. What is QoS (Quality of Service)?

Ans: QoS is a router feature that gives priority to important traffic. Example: Video calls get priority over file downloads, so your call stays clear even if someone is downloading a movie.

Q51. What is WPA2/WPA3?

Ans: WPA2 and WPA3 are Wi-Fi security protocols. They encrypt your Wi-Fi connection so others cannot read your data. WPA3 is newer and more secure than WPA2.

Q52. What is MAC address filtering?

Ans: MAC address filtering allows only specific devices to connect to a network. Each device has a unique MAC address. The router checks this address and blocks unknown devices.

SECTION 6: Quick Fire — One Line Answers

| No. | Question | Answer |
| --- | --- | --- |
| Q53 | What does IP stand for? | Internet Protocol |
| Q54 | What does DHCP stand for? | Dynamic Host Configuration Protocol |
| Q55 | What does DNS stand for? | Domain Name System — converts website names to IP addresses |
| Q56 | What is 127.0.0.1? | Loopback address — used to test your own network card |
| Q57 | What is 255.255.255.255? | Limited broadcast address — sends data to all devices on the network |
| Q58 | What is the subnet mask for /24? | 255.255.255.0 |
| Q59 | What is the subnet mask for /26? | 255.255.255.192 |
| Q60 | What is the subnet mask for /28? | 255.255.255.240 |
| Q61 | How many bits in IPv4? | 32 bits |
| Q62 | How many bits in IPv6? | 128 bits |
| Q63 | What does COA stand for? | Care-of-Address — temporary IP in foreign network |
| Q64 | What does HA stand for? | Home Agent |
| Q65 | What does FA stand for? | Foreign Agent |
| Q66 | What does MN stand for? | Mobile Node |
| Q67 | What is IP-in-IP tunneling? | Wrapping one IP packet inside another to forward it to a different network |
| Q68 | What does SSID stand for? | Service Set Identifier — the name of a Wi-Fi network |
| Q69 | What command tests connectivity? | ping <IP address> |
| Q70 | What is the difference between /24 and /26? | /24 has 254 hosts, /26 has 62 hosts. /26 is a smaller subnet. |
| Q71 | What is RFC 3344? | The standard document that defines Mobile IP for IPv4 |
| Q72 | What is Triangle Routing? | When data goes from Sender to Home Agent to Foreign Agent to Mobile Node — an indirect path |

SECTION 7: Important Flows to Remember

Mobile IP Registration Flow:

| Step | Who Acts | What Happens |
| --- | --- | --- |
| 1 | Mobile Node (MN) | Moves to foreign network, receives Agent Advertisement from FA |
| 2 | Mobile Node (MN) | Gets a Care-of-Address (COA) from FA |
| 3 | Mobile Node → FA | Sends Registration Request (contains Home IP, HA address, COA) |
| 4 | FA → Home Agent | Forwards Registration Request to HA |
| 5 | Home Agent | Verifies MN identity, updates binding table (Home IP → COA) |
| 6 | HA → FA → MN | Sends Registration Reply (success/failure) |
| 7 | Home Agent | Now tunnels all packets for MN to the new COA |

Subnetting Quick Formula:

| What to Find | Formula | Example (/26) |
| --- | --- | --- |
| Subnet bits | New prefix - Class default | 26 - 24 = 2 |
| Number of subnets | 2^(subnet bits) | 2^2 = 4 subnets |
| Host bits | 32 - prefix length | 32 - 26 = 6 |
| Hosts per subnet | 2^(host bits) - 2 | 2^6 - 2 = 62 hosts |
| Block size | 256 - last octet of mask | 256 - 192 = 64 |

CIDR to Subnet Mask Reference:

| CIDR | Subnet Mask | Subnets (from /24) | Hosts/Subnet |
| --- | --- | --- | --- |
| /24 | 255.255.255.0 | 1 | 254 |
| /25 | 255.255.255.128 | 2 | 126 |
| /26 | 255.255.255.192 | 4 | 62 |
| /27 | 255.255.255.224 | 8 | 30 |
| /28 | 255.255.255.240 | 16 | 14 |
| /29 | 255.255.255.248 | 32 | 6 |
| /30 | 255.255.255.252 | 64 | 2 |

IT154MJP Advanced Networking | Viva Q&A | F.Y.B.Sc.(IT) Sem-II NEP | SPPU 2026